Georgia Institute of Technology

The George W. Woodruff School of Mechanical Engineering

Nuclear and Radiological Engineering/Medical Physics Program

PhD Qualifying Exam

Fall 2019

___________

(Your ID Code)

Nuclear Engineering

Reactor Physics

(Day 2)

Instructions

- Use a separate page for each answer sheet using only the front side of the paper.  DO NOT write on the back of the answer sheet

- The question number and your ID Code should be shown clearly on each answer sheet

- ANSWER 4 OF 6 Questions in this exam

- Staple your question sheet to your answer sheet and turn in

Reactor Physics Question 1

A core fueled with 100 ton (4.5% weight enriched) of uranium operates under a nominal power of 3400 MWth.   The following simplified transmutation chain is given:

fission→Pm149β-→t1/2=53 hr Sm149(n,γ)→σ=40×103 b

In addition, the fission yield for Pm is γPm=0.0113. The half-life of Sm149 is extremely long and the cross-section of Pm149 is negligible. Assume that U235 is the only fissile isotope with an effective one-group cross-section of σf=30 b. Assume the nuclide density of U235 is not changing.

- Write the differential equations that describe the rate of change for Pm149 and Sm149.

- Calculate the concentration (# of atoms) of Sm149 at equilibrium.

- Calculate the concentration of Sm149 following a shutdown of 20 days.

- Draw the reactivity worth shape of Sm149 following a shutdown and explain/justify your drawing

Reactor Physics Question 2

The power of a core is required to be doubled in 5 minutes. Before the transient, the core operated at a steady state with a nominal power of P0.

- Calculate the stable period.

- Calculate the inserted reactivity to satisfy this increase.

Data: =0.08 s-1 , β=0.0065

Reactor Physics Question 3

A bare homogeneous spherical core produces 15 MWth. The radius of the core is 100 cm. Calculate the power density (W/cm3) of the core at the point z=7 cm, y=4 cm, z=-22 cm. The coordinate’s origin is positioned at the center the spherical core. Neglect the extrapolation length.

Reactor Physics Question 4

Using diffusion theory, compute the buckling of a critical, 150 MWth, fast reactor made of mixed oxide fuel disbursed uniformly in a 10-cm by 10-cm by 20-cm configuration. Assume that υΣf=0.2 cm-1 and the transport mean free path λtr at all surfaces of this reactor is 4-cm.

Reactor Physics Question 5

Consider a homogeneous slab consisting of uranium-235, the neutrons are essentially all fast (E≥100  kev) and, as a first approximation, all the neutrons may be considered to have the same energy.

- Write down the diffusion equation, assuming isotropic scattering and vacuum boundary condition.

- Calculate the critical thickness. The following data are to be used: uranium density 10.88 g/cm3,  σf=1.3 barns, σs=4.0 barns, σc=0, and ν=2.5.

Reactor Physics Question 6

For a homogeneous purely scattering infinity medium, there is a source sz,E=SE for z∈(-∞,∞).

  - Write down the neutron slowing down equation, assuming the scattering cross section is ΣsE'→E,Ω'→Ω=14πE' for E∈[0,E'].

  - Derive the lethargy (u) dependent slowing down equation from (a)