Georgia Institute of Technology

The George W. Woodruff School of Mechanical Engineering

Nuclear & Radiological Engineering/Medical Physics Program

Ph.D. Qualifier Exam

Fall Semester 2017

______________

Your ID Code

NRE Fusion

(Day 2)

Instructions

- Use a separate page for each answer sheet using only one side of the paper.   DO NOT write on the BACK of the answer sheet.

- The question number and your ID Code should be shown clearly on each answer sheet

3.  ANSWER 4 OF 6 QUESTIONS ONLY

4.  Staple your question sheet to your answer sheets and turn in

NRE  Fusion

Answer any 4 of the following 6 questions

Question 1:

It is generally desirable to have as large a value of energy confinement in a plasma as possible.  In a tokamak the energy confinement time depends strongly on the plasma current, among other things.  Discuss how the requirement for MHD kink mode stability sets a lower limit on the toroidal magnetic field strength required for a given plasma current.  Calculate the minimum toroidal magnetic field B÷ that is required for a plasma current of I=5 MA in a circular cross section tokamak of Minor Radius r=a = 1.5m and major radius of R = 4 m.

Question 2:

Describe how you would calculate the D-T fusion power density in the tokamak of problem 1.  What physics limits would need be taken into account in determining the maximum achievable power density?

Question 3:

Discuss ion orbit loss of thermalized ions in the outflowing plasma.  What is it? How is it calculated?  How does it affect the plasma parameters?

Question 4:

How are plasmas heated to thermonuclear temperatures?  Discuss the physics of the principal heating mechanisms.

Question 5:

Discuss the physics processes involved in the plasma-wall interaction between the very energetic plasma and the material wall.

Question 6:

Describe how the plasma fluid equations are derived from the Boltzmann kinetic transport equations that describe the particle distribution functions.