Georgia Institute of Technology

The George W. Woodruff School of Mechanical Engineering

Nuclear & Radiological Engineering/Medical Physics Program

Ph.D. Qualifier Exam

Fall Semester 2017

______________

Your ID Code

NRE Reactor Physics

(Day 2)

Instructions

- Use a separate page for each answer sheet using only one side of the paper.   DO NOT write on the BACK of the answer sheet.

- The question number and your ID Code should be shown clearly on each answer sheet.

3.  ANSWER 4 OF 6 QUESTIONS ONLY.

4.  Staple your question sheet to your answer sheets and turn in.

NRE  Reactor Physics

Answer any 4 of the following 6 questions

Question 1:

Calculate the neutron energy spectrum ϕE from a mono-energetic source emitting neutrons at E0 energy in an infinite hydrogen medium. Assume that the scattering cross section is constant and the absorption cross-section is inversely proportional to the neutron velocity.

Question 2:

Iodine (I) and Xenon (Xe) are produced in fission events. Iodine decays to Xenon which subsequently can either decay or absorb a neutron.

- What should be the maximum value of the flux such that there will be no increase in Xenon concentration following shut-down. Explain why this is happening.

- The reactor operates with a nominal power. If the power increases what will happen to the Xenon reactivity worth.

Use the following data:

σaXe=2.6×106 b, t1/2I=6.6 h,  t1/2Xe=9.1 h, γI=0.061, γXe=0.003, ν=2.46

Question 3:

The power in a reactor is doubled in 5 minutes by inserting a fixed positive reactivity. Calculate the stable period of the reactor and the inserted reactivity value. β=0.0065, λ=0.08 sec-1 and Λ=10-3 sec.

Question 4:

A bare 40 cm slab reactor with homogeneously distributed fuel is given. The group constants are given in the following table:

|  | Group 1 | Group 2 | Group 3 |
| --- | --- | --- | --- |
| χ | 0.580 | 0.430 | 0.000 |
| νΣf, cm-1 | 0.010 | 0.020 | 0.180 |
| Σa, cm-1 | 0.005 | 0.030 | 0.120 |
| Σsg⟶g+1, cm-1 | 0.083 | 0.124 | 0.000 |
| D, cm-1 | 2.160 | 1.720 | 0.350 |

Calculate the neutron energy spectrum (in 3 groups) and the criticality value. Assume that there is no up-scattering and only scattering to adjacent groups is possible. Describe your solution in details.

Question 5:

A spherical reactor composed of 235U metal is operating in a critical steady state. Discuss what probably happens to the multiplication of the reactor and why, if the system is modified in the following ways (treat each modification separately, not cumulatively):

a. The reactor is rapidly compressed to one-half its original volume.

b. A reactor operator accidentally sits on the reactor, squashing it into an ellipsoidal shape.

c. A thick sheet of cadmium is wrapped around the outside of the reactor. (Cadmium has large thermal absorption cross section).

d. The reactor is suddenly immersed in a large container of water.

e. A source of neutrons is placed near the reactor.

f. Another identical reactor is placed a short distance from the original reactor.

g. One simply leaves the reactor alone for a period of time.

Question 6:

A critical reactor at a reference state may be described with a loss operator, L, and production operator, P, as:

[L – λP] Φ = 0

A perturbed state is described by:

[L’ – λ’P’] Φ’ = 0

where

L’ = L + ΔL

λ’ = λ + Δλ

P’ = P + ΔP

Φ’ = Φ + ΔΦ

Develop a perturbation theory expression for Δλ and show that the adjoint flux Φ+ reduces the error to second order.